Short sequences

| Data Field | Element Name | Descrip. | Table B Scale | Table B Ref. Val. | Table B Width | Units | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1–4 | WIGOS identifier | 3 01 150 |  |  |  |  |  |
| 1 | Identifier series | 0 01 125 | 0 | 0 | 4 | Numeric |  |
| 2 | Issuer of identifier | 0 01 126 | 0 | 0 | 16 | Numeric |  |
| 3 | Issue number | 0 01 127 | 0 | 0 | 16 | Numeric |  |
| 4 | Local identifier | 0 01 128 | 0 | 0 | 128 | Character |  |

| Data Field | Element Name | Descrip. | Table B Scale | Table B Ref. Val. | Table B Width | Units | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1–3 | Radio occultation satellite data introducer | New |  |  |  |  |  |
| 1 | Satellite Identifier | 0 01 007 | 0 | 0 | 10 | Code Table | LEO or LEO constellation – See Table 1 |
| 2 | Satellite sub-identifier | 0 01 016 | 0 | 0 | 16 | Numeric | Satellite ID in LEO constellation (missing if not applicable) |
| 3 | Satellite instrument | 0 02 019 | 0 | 0 | 11 | Code Table | e.g. 202 = GRAS – See Table 2 |

| Data Field | Element Name | Descrip. | Table B Scale | Table B Ref. Val. | Table B Width | Units | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1–3 | Aircraft data introducer | New |  |  |  |  | To be proposed by the first provider of such data, e.g., Scripps Institute of Oceanography – the below is tentative until then |
| 1 | Aircraft registration number or other identification | 0 01 008 | 0 | 0 | 64 | Character |  |
| 2 | Aircraft tail number | 0 01 110 | 0 | 0 | 48 | Character |  |
| 3 | Upper air remote sensing instrument type | 0 02 006 | 0 | 0 | 6 | Code Table |  |

| Data Field | Element Name | Descrip. | Table B Scale | Table B Ref. Val. | Table B Width | Units | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1–3 | Balloon data introducer | New |  |  |  |  | To be proposed by the first provider of such data, e.g., Scripps Institute of Oceanography – the below is tentative until then |
| 1 | Balloon Registration number or other identification | 0 01 157 | 0 | 0 | 80 | Character | On Fast-Track FT2025-2 here: https://community.wmo.int/site/knowledge-hub/programmes-and-initiatives/wmo-information-system-wis/about-manual-codes-volume-i2 |
| 2 | Balloon flight number | 0 01 156 | 0 | 0 | 64 | Character | On Fast-Track FT2025-2 here: https://community.wmo.int/site/knowledge-hub/programmes-and-initiatives/wmo-information-system-wis/about-manual-codes-volume-i2 |
| 3 | Upper air remote sensing instrument type | 0 02 006 | 0 | 0 | 6 | Code Table |  |

| Data Field | Element Name | Descrip. | Table B Scale | Table B Ref. Val. | Table B Width | Units | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1–2 | Station data introducer | New |  |  |  |  | To be proposed by the first provider of such data – the below is tentative until then |
| 1 | Short station or site name | 0 01 018 | 0 | 0 | 40 | Character |  |
| 2 | Upper air remote sensing instrument type | 0 02 006 | 0 | 0 | 6 | Code Table |  |

BUFR Section 3 (Data Description)

| Octet number | Contents | Value for RO |
| --- | --- | --- |
| 1–3 | Length of Section 3 (octets) | 13 or 14 |
| 4 | Reserved | 0 |
| 5–6 | Number of datasets (observations) | Integer > 0 |
| 7 | Section 4 Data flags: Bit 1 = 1 for Observed data, 0 for Other data Bit 2 = 1 for Compressed, 0 for Uncompressed Bits 3–8: Reserved (set to 0) | 128 (Observed, Uncompressed) or 192 (Observed, Compressed) |
| 8–9 | Descriptor (WIGOS identifier) | 49558 (3 x 16384 + 1 x 256 + 150) |
| 10–11 | Descriptor (e.g., new Aircraft data introducer) | New (3 x 16384 + X x 256 + Y) |
| 12–13 | Descriptor (new RO BUFR template) | New (3 x 16384 + 10 x 256 + Y) |
| [14–] | Repeat 2-octets per descriptor Total number of descriptors = (Section length - 7) / 2 |  |
| [14] | Pad byte (set to 0) – optional for Edition 4 | 0 (if Section length = 14) |

The above table is an example of how Section 3 in a BUFR message could look, including the WIGOS identifier and the data introducer before the new RO BUFR template.

New RO BUFR template

| Data Field | Element Name | Descrip. | Table B Scale | Table B Ref. Val. | Table B Width | Units | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Radio Occultation header |  |  |  |  |  |  |  |
| 1 | Originating centre | 0 01 035 | 0 | 0 | 16 | Code Table | e.g. 94 = Copenhagen (DMI) |
| 2 | Originating sub-centre | 0 01 034 | 0 | 0 | 8 | Code Table | e.g. 220 = NOAA |
| 3 | Product type | 0 02 172 | 0 | 0 | 8 | Code Table | 3 = occultation sounding |
| 4 | Software ID and version | 0 25 061 | 0 | 0 | 96 | Character | Identifies processing algorithms used by org. centre |
| 5 | Processing mode and disposition | 0 25 new | 0 | 0 | 5 | Code Table | e.g. 3 = Reprocessing – See Table 3 |
| Reference time |  |  |  |  |  |  |  |
| 6 | Time Significance | 0 08 021 | 0 | 0 | 5 | Code Table | 17 = start of phenomenon or 25 = nominal reporting time |
| 7–9 | Date | 3 01 011 |  |  |  |  |  |
| 7 | Year | 0 04 001 | 0 | 0 | 12 | Year | 4-digit year |
| 8 | Month | 0 04 002 | 0 | 0 | 4 | Month |  |
| 9 | Day | 0 04 003 | 0 | 0 | 6 | Day |  |
| 10–11 | Time | 3 01 012 |  |  |  |  |  |
| 10 | Hour | 0 04 004 | 0 | 0 | 5 | Hour |  |
| 11 | Minute | 0 04 005 | 0 | 0 | 6 | Minute |  |
|  | Change Table B bit width | 2 01 138 |  |  |  |  | Add 10 to bit width |
|  | Change Table B scale | 2 02 131 |  |  |  |  | Add 3 to scale |
| 12 | Second | 0 04 006 | (3) 0 | 0 | (16) 6 | Second | 0.000 – 59.999s to 1ms |
|  | Change scale to Table B | 2 02 000 |  |  |  |  |  |
|  | Change width to Table B | 2 01 000 |  |  |  |  |  |
| RO summary quality information |  |  |  |  |  |  |  |
| 13 | Quality flags for RO Data | 0 33 039 | 0 | 0 | 16 | Flag table | See Table 4 |
| 14 | Percent confidence | 0 33 007 | 0 | 0 | 7 | % | 0 = bad, 100 = good |
| LEO & GNSS POD at reference time |  |  |  |  |  |  |  |
| 15 | Reference system | 0 01 150 | 0 | 0 | 16 | Code Table | 5 = ECEF or ECR, right-handed |
| 16–18 | Location of Platform (see Note 10) | 3 04 030 |  |  |  |  | LEO, to 1cm |
| 16 | Distance from Earth’s centre in direction 0o longitude | 0 27 031 | 2 | -1073741824 | 31 | Metres | (X) |
| 17 | Distance from Earth’s centre in direction 90o east longitude | 0 28 031 | 2 | -1073741824 | 31 | Metres | (Y) |
| 18 | Distance from Earth’s centre in direction of North Pole | 0 10 031 | 2 | -1073741824 | 31 | Metres | (Z) |
| 19–21 | Velocity of Platform (see Note 10) | 3 04 031 |  |  |  |  | LEO, to 10mm/s |
| 19 | Absolute platform velocity – first component. | 0 01 041 | 5 | -1073741824 | 31 | Metres per second | (X) |
| 20 | Absolute platform velocity – second component. | 0 01 042 | 5 | -1073741824 | 31 | Metres per second | (Y) |
| 21 | Absolute platform velocity – third component. | 0 01 043 | 5 | -1073741824 | 31 | Metres per second | (Z) |
| 22 | Satellite Classification | 0 02 020 | 0 | 0 | 9 | Code Table | GNSS series, e.g. 401=GPS – See Table 5 |
| 23 | Platform Transmitter ID | 0 01 050 | 0 | 0 | 17 | Numeric | GNSS PRN or similar |
|  | Change Table B scale | 2 02 127 |  |  |  |  | Subtract 1 from scale |
| 24–26 | Location of Platform (see Note 10) | 3 04 030 |  |  |  |  | GNSS, to 10cm |
| 24 | Distance from Earth’s centre in direction 0o longitude | 0 27 031 | (1) 2 | -1073741824 | 31 | Metres | (X) |
| 25 | Distance from Earth’s centre in direction 90o east longitude | 0 28 031 | (1) 2 | -1073741824 | 31 | Metres | (Y) |
| 26 | Distance from Earth’s centre in direction of North Pole | 0 10 031 | (1) 2 | -1073741824 | 31 | Metres | (Z) |
|  | Change scale to Table B | 2 02 000 |  |  |  |  |  |
| 27–29 | Velocity of Platform (see Note 10) | 3 04 031 |  |  |  |  | GNSS, to 10mm/s |
| 27 | Absolute platform velocity – first component. | 0 01 041 | 5 | -1073741824 | 31 | Metres per second | (X) |
| 28 | Absolute platform velocity – second component. | 0 01 042 | 5 | -1073741824 | 31 | Metres per second | (Y) |
| 29 | Absolute platform velocity – third component. | 0 01 043 | 5 | -1073741824 | 31 | Metres per second | (Z) |
| Local Earth parameters at reference time |  |  |  |  |  |  |  |
| 30 | Straight line tangent altitude | 0 10 new | 0 | -400000 | 19 | Metres | -400 – 124km wrt WGS-84 |
| 31–32 | Location (high accuracy) | 3 01 021 |  |  |  |  |  |
| 31 | Latitude | 0 05 001 | 5 | -9000000 | 25 | Degrees | to 10-5 deg (~1m) wrt WGS-84 |
| 32 | Longitude | 0 06 001 | 5 | -18000000 | 26 | Degrees | to 10-5 deg (~1m) wrt WGS-84 |
| 33–35 | Location of point | 3 04 030 |  |  |  |  | (X, Y, Z) centre of curvature to 1cm |
| 33 | Distance from Earth’s centre in direction 0o longitude | 0 27 031 | 2 | -1073741824 | 31 | Metres | (X) |
| 34 | Distance from Earth’s centre in direction 90o east longitude | 0 28 031 | 2 | -1073741824 | 31 | Metres | (Y) |
| 35 | Distance from Earth’s centre in direction of North pole | 0 10 031 | 2 | -1073741824 | 31 | Metres | (Z) |
| 36 | Earth’s local radius of curvature | 0 10 035 | 1 | 62 000 000 | 22 | Metres | RoC of WGS-84 ellipsoid. 6200–6600 km to 10cm |
| 37 | Bearing or azimuth | 0 05 021 | 2 | 0 | 16 | Deg. True | GNSS-to-LEO line of sight |
| 38 | Geoid undulation | 0 10 036 | 2 | -15 000 | 15 | Metres | Geoid height above WGS-84 ellipsoid. ±150m to 1cm |
| Time period of occultation |  |  |  |  |  |  |  |
| 39 | Time period (between reference time and start of occultation) | 0 04 026 | 0 | -4096 | 13 | Second | -4096s – 4096s (normally negative) |
| 40 | Time period (between reference time and end of occultation) | 0 04 026 | 0 | -4096 | 13 | Second | -4096s – 4096s (normally positive) |
| Diagnostic quality information |  |  |  |  |  |  |  |
|  | Delayed Replication | 1 06 000 |  |  |  |  | Delayed replication of next 6 descriptors |
| 41 | Replication factor | 0 31 001 | 0 | 0 | 8 | Numeric | Number of height ranges, n0 |
| 42 | Height (bottom of range) | 0 07 007 | 0 | -1000 | 17 | Metres | -1km – 130km, to 1m |
| 43 | Height (top of range) | 0 07 007 | 0 | -1000 | 17 | Metres | -1km – 130km, to 1m |
|  | Change Table B scale | 2 02 134 |  |  |  |  | Add 6 to scale |
| 44 | Satellite channel centre frequency | 0 02 153 | (-2) -8 | 0 | 26 | Hz | 0 – 6.7GHz, to 0.1kHz |
|  | Change scale to Table B | 2 02 000 |  |  |  |  |  |
| 45 | Signal-to-noise ratio, mean value over specified height range | 0 33 new | 1 | 0 | 20 | Numeric | 0 – 100000, to 0.1 |
| N0 | (end delayed replication – number of height ranges) |  |  |  |  |  | N0 = 41+4.n0 elements |
|  | Delayed Replication | 1 06 000 |  |  |  |  | Delayed replication of next 6 descriptors |
| N0+1 | Replication factor | 0 31 001 | 0 | 0 | 8 | Numeric | Number of height ranges, n1 |
| N0+2 | Height (bottom of range) | 0 07 007 | 0 | -1000 | 17 | Metres | -1km – 130km, to 1m |
| N0+3 | Height (top of range) | 0 07 007 | 0 | -1000 | 17 | Metres | -1km – 130km, to 1m |
|  | Change Table B scale | 2 02 134 |  |  |  |  | Add 6 to scale |
| N0+4 | Satellite channel centre frequency | 0 02 153 | (-2) -8 | 0 | 26 | Hz | 0 – 6.7GHz, to 0.1kHz, Ionosphere corrected = 0 |
|  | Change scale to Table B | 2 02 000 |  |  |  |  |  |
| N0+5 | Bending angle noise, standard deviation over specified height range | 0 33 new | 8 | 0 | 16 | Radians | 0 – 6x10-4 rad, to 10-8 rad |
| N1 | (end delayed replication – number of height ranges) |  |  |  |  |  | N1 = N0+1+4.n1 elements |
| N1+1 | Satellite channel centre frequency | 0 02 153 | -8 | 0 | 26 | Hz | Missing = cancel previous value |
| N1+2 | Height | 0 07 007 | 0 | -1000 | 17 | Metres | Missing = cancel previous value |
| Characteristic heights |  |  |  |  |  |  |  |
|  | Delayed Replication | 1 03 000 |  |  |  |  | Delayed replication of next 3 descriptors |
| N1+3 | Replication factor | 0 31 001 | 0 | 0 | 8 | Numeric | Number of characteristic heights, n2 |
| N1+4 | Radio occultation vertical significance | 0 08 new | 0 | 0 | 6 | Code Table | See Table 7 |
| N1+5 | Height | 0 10 007 | 0 | -1000 | 17 | Metres | -1km – 130km, to 1m |
| N1+6 | Standard deviation altitude | 0 10 050 | 2 | 0 | 16 | Metres | 0 – 655m, to 1cm |
| N2 | (end delayed replication – number of characteristic heights) |  |  |  |  |  | N2 = N1+3+3.n2 elements |
| N2+1 | Radio occultation vertical significance | 0 08 new | 0 | 0 | 6 | Code Table | Missing = cancel previous value |
| RO 'Step 1b' data (see Notes 2 & 5) |  |  |  |  |  |  |  |
|  | Delayed replication | 1 26 000 |  |  |  |  | Delayed replication of next 26 descriptors |
| N2+2 | Replication factor | 0 31 002 | 0 | 0 | 16 | Numeric | Number of Step 1b samples, n6 |
| N2+3–N2+4 | Location (high accuracy) | 3 01 021 |  |  |  |  |  |
| N2+3 | Latitude | 0 05 001 | 5 | -9000000 | 25 | Degrees | to 10-5 deg (~1m) wrt WGS-84 |
| N2+4 | Longitude | 0 06 001 | 5 | -18000000 | 26 | Degrees | to 10-5 deg (~1m) wrt WGS-84 |
| N2+5 | Bearing or azimuth | 0 05 021 | 2 | 0 | 16 | Deg. True | GNSS-to-LEO line of sight |
| N2+6 | Impact Parameter | 0 07 040 | 1 | 62 000 000 | 22 | Metres | 6200–6600 km to 10cm (distance from centre of curvature) |
|  | Delayed Replication | 1 11 000 |  |  |  |  | Delayed replication of next 11 descriptors |
| N2+7 | Replication factor | 0 31 001 | 0 | 0 | 8 | Numeric | Number of frequencies, n4 |
|  | Change Table B scale | 2 02 134 |  |  |  |  | Add 6 to scale |
| N2+8 | Satellite channel centre frequency | 0 02 153 | (-2) -8 | 0 | 26 | Hz | 0 – 6.7GHz, to 0.1kHz, Ionosphere corrected = 0 |
|  | Change scale to Table B | 2 02 000 |  |  |  |  |  |
| N2+9 | Signal type | 0 02 new | 0 | 0 | 24 | Character | Three-letter code, RINEX convention |
| N2+10 | Bending Angle | 0 15 037 | 8 | -100 000 | 23 | Radians | -10-3 – 8x10-2 rad to 10-8 rad |
| N2+11 | Estimated error in bending angle | 0 15 new | 8 | 0 | 20 | Radians | 0 – 10-2 rad to 10-8 rad |
| N2+12 | Calibrated phase difference | 0 15 new | 5 | -5000 | 14 | Metres | -50mm – 113mm to 0.01mm |
| N2+13 | Estimated error in calibrated phase difference | 0 15 new | 5 | 0 | 11 | Metres | 0 – 20mm to 0.01mm |
|  | Delayed Replication | 1 01 000 |  |  |  |  | Delayed replication of next descriptor |
| N2+14 | Short delayed replication factor | 0 31 000 | 0 | 0 | 1 | Numeric | Indicate presence of aircraft or station RO data, n3 |
| N2+15 – N2+18 | Aircraft or station RO data | 3 12 new |  |  |  |  |  |
|  | Increase Table B scale, reference value and bit width (all at once) | 2 07 001 |  |  |  |  | Add 1 to scale, multiply reference value by 10, and add 4 to bit width |
| N2+15 | Height (of RO antenna) | 0 10 007 | (1) 0 | (-10000) -1000 | (21) 17 | Metres | Geom. altitude, -1km – 200km, to 10cm, wrt Geoid (MSL) |
|  | Change scale, reference value and bit width to Table B | 2 07 000 |  |  |  |  |  |
| N2+16 | Atmospheric refractivity (in-situ at RO antenna) | 0 15 036 | 3 | 0 | 19 | N-units | 0 – 500 to 10-3 N-units |
| N2+17 | Bending angle (partial) | 0 15 037 | 8 | -100 000 | 23 | Radians | -10-3 – 8x10-2 rad to 10-8 rad |
| N2+18 | Estimated error in bending angle | 0 15 new | 8 | 0 | 20 | Radians | 0 – 10-2 rad to 10-8 rad |
| N3 | (end delayed replication – presence of aircraft or station RO data) |  |  |  |  |  | N3 = N2+14+4.n3 elements |
| N4 | (end delayed replication – number of frequencies) |  |  |  |  |  | N4 = N2+7+(7+4.n3).n4 elements |
| N4+1 | Signal type | 0 02 new | 0 | 0 | 24 | Character | Missing = cancel previous value |
| N4+2 | Satellite channel centre frequency | 0 02 153 | -8 | 0 | 26 | Hz | 0 = Ionosphere corrected |
| N4+3 | Percent confidence | 0 33 007 | 0 | 0 | 7 | % | 0 = bad, 100 = good |
|  | Delayed Replication | 1 05 000 |  |  |  |  | Delayed replication of next 5 descriptors |
| N4+4 | Replication factor | 0 31 001 | 0 | 0 | 8 | Numeric | Number of diagnostic entries, n5 |
| N4+5 | Radio occultation Diagnostic type | 0 08 new | 0 | 0 | 5 | Code Table | See Table 6 |
| N4+6 | Decimal scale of following significands | 0 08 090 | 0 | -127 | 8 | Numeric | Decimal scale (see next descriptor) |
| N4+7 | Scaled diagnostic | 0 33 new | 0 | -131 071 | 18 | Numeric | Scaled value (actual value = scaled value x 10decimal scale) |
| N4+8 | Decimal scale of following significands | 0 08 090 | 0 | -127 | 8 | Numeric | Missing = cancel previous value |
| N4+9 | Radio occultation Diagnostic type | 0 08 new | 0 | 0 | 5 | Code Table | Missing = cancel previous value |
| N5 | (end delayed replication – number of diagnostic entries) |  |  |  |  |  | N5 = N4+4+5.n5 elements |
| N6 | (end delayed replication – number of step 1B samples) |  |  |  |  |  | N6 = N2+2+(9+(7+4.n3).n4+5.n5).n6 elements |
| N6+1 | Impact Parameter | 0 07 040 | 1 | 62 000 000 | 22 | Metres | Missing = cancel previous value |
| N6+2 | Bearing or azimuth | 0 05 021 | 2 | 0 | 16 | Deg. True | Missing = cancel previous value |
| N6+3–N6+4 | Location (high accuracy) | 3 01 021 |  |  |  |  |  |
| N6+3 | Latitude | 0 05 001 | 5 | -9000000 | 25 | Degrees | Missing = cancel previous value |
| N6+4 | Longitude | 0 06 001 | 5 | -18000000 | 26 | Degrees | Missing = cancel previous value |
| RO 'Step 2a' data (see Notes 3–5 & 11) |  |  |  |  |  |  |  |
|  | Delayed replication | 1 15 000 |  |  |  |  | Delayed replication of next 15 descriptors |
| N6+5 | Replication factor | 0 31 002 | 0 | 0 | 16 | Numeric | Number of Step 2a samples, n8 |
|  | Increase Table B scale, reference value and bit width (all at once) | 2 07 001 |  |  |  |  | Add 1 to scale, multiply reference value by 10, and add 4 to bit width |
| N6+6 | Height | 0 07 007 | (1) 0 | (-10000) -1000 | (21) 17 | Metres | Geom. altitude, -1km – 200km, to 10cm, wrt Geoid (MSL) |
| N6+7 | Geopotential height (see Note 11) | 0 10 009 | (1) 0 | (-10000) -1000 | (21) 17 | gpm | Geopot. altitude, -1km – 200km, to 10cm, wrt Geoid (MSL) |
| N6+8 | Atmospheric refractivity | 0 15 036 | (4) 3 | 0 | (23) 19 | N-units | 0 – 800, to 10-4 N-units |
| N6+9 | Estimated error in atmospheric refractivity | 0 15 new | (4) 3 | 0 | (20) 16 | N-units | 0 – 100, to 10-4 N-units |
| N6+10 | Dry temperature | 0 12 new | (2) 1 | 0 | (16) 12 | K | 0 – 600K, to 0.01 K |
|  | Change scale, reference value and bit width to Table B | 2 07 000 |  |  |  |  |  |
| N6+11 | Percent confidence | 0 33 007 | 0 | 0 | 7 | % | 0 = bad, 100 = good |
|  | Delayed Replication | 1 04 000 |  |  |  |  | Delayed replication of next 4 descriptors |
| N6+12 | Replication factor | 0 31 001 | 0 | 0 | 8 | Numeric | Number of diagnostic entries, n7 |
| N6+13 | Radio occultation Diagnostic type | 0 08 new | 0 | 0 | 5 | Code Table | See Table 6 |
| N6+14 | Decimal scale of following significands | 0 08 090 | 0 | -127 | 8 | Numeric | Decimal scale (see next descriptor) |
| N6+15 | Scaled diagnostic | 0 33 new | 1 | -131 071 | 18 | Numeric | Scaled value (actual value = scaled value x 10decimal scale) |
| N6+16 | Decimal scale of following significands | 0 08 090 | 0 | -127 | 8 | Numeric | Missing = cancel previous value |
| N6+17 | Radio occultation Diagnostic type | 0 08 new | 0 | 0 | 5 | Code Table | Missing = cancel previous value |
| N7 | (end delayed replication – number of diagnostics entries) |  |  |  |  |  | N7 = N6+12+5.n7 elements |
| N8 | (end delayed replication – number of step 2a samples) |  |  |  |  |  | N8 = N6+5+(7+5.n7).n8 elements |
| Blue entries indicate new elements. |  |  |  |  |  |  |  |

Table 1. GNSS RO Satellite Identifier (Common Code Table C–5)

| Descriptor | Value | Name | Value | Name |
| --- | --- | --- | --- | --- |
| 0 01 007 | 3 | Metop-1 (Metop-B) | 740 | COSMIC-1 |
|  | 4 | Metop-2 (Metop-A) | 741 | COSMIC-2 |
|  | 5 | Metop-3 (Metop-C) | 742 | COSMIC-3 |
|  | 40 | Ørsted (or Oersted) | 743 | COSMIC-4 |
|  | 41 | CHAMP | 744 | COSMIC-5 |
|  | 42 | TerraSAR-X | 745 | COSMIC-6 |
|  | 43 | TanDEM-X | 750 | COSMIC-2 E1 |
|  | 44 | PAZ | 751 | COSMIC-2 E2 |
|  | 66 | Sentinel-6A | 752 | COSMIC-2 E3 |
|  | 67 | Sentinel-6B | 753 | COSMIC-2 E4 |
|  | 265 | GeoOptics CICERO OP1 | 754 | COSMIC-2 E5 |
|  | 266 | GeoOptics CICERO OP2 | 755 | COSMIC-2 E6 |
|  | 267 | PlanetiQ GNOMES-A | 786 | C/NOFS |
|  | 268 | PlanetiQ GNOMES-B | 800 | SUNSAT |
|  | 269 | Spire Lemur 3U CubeSat | 803 | GRACE C (GRACE-FO) |
|  | 421 | Oceansat-2 | 804 | GRACE D (GRACE-FO) |
|  | 440 | Megha-Tropiques | 820 | SAC-C |
|  | 522 | FY-3C | 825 | KOMPSAT-5 |
|  | 523 | FY-3D |  |  |
|  | 722 | GRACE A |  |  |
|  | 723 | GRACE B |  |  |

Table 2. GNSS RO Satellite Instrument Identifier (Common Code Table C–8)

| Descriptor | Value | Type/longname (shortname) |
| --- | --- | --- |
| 0 02 019 | 98 | Atmospheric temperature and humidity sounder (GPS receiver) |
|  | 102 | GPS turborogue space receiver (TRSR) (CHAMP GPS sounder) |
|  | 103 | Integrated GPS and Occultation Receiver (IGOR) |
|  | 104 | Triple-G (GPS, Galileo, GLONASS) (Tri-G) |
|  | 202 | GNSS receiver for atmospheric sounding (GRAS) |
|  | 287 | Radio Occultation Sounder of the Atmosphere (ROSA) |
|  | 351 | GPS demonstration receiver (GPSDR) |
|  | 526 | GeoOptics Cion GNSS occultation receiver A (Cion-A) |
|  | 527 | GeoOptics Cion GNSS occultation receiver B (Cion-B) |
|  | 528 | GeoOptics Cion GNSS occultation receiver C (Cion-C) |
|  | 529 | GeoOptics Cion GNSS occultation receiver D (Cion-D) |
|  | 530 | Spire global navigation satellite system occultation sounder A (SGNOS-A) |
|  | 531 | Spire global navigation satellite system occultation sounder B (SGNOS-B) |
|  | 532 | Spire global navigation satellite system occultation sounder C (SGNOS-C) |
|  | 533 | Spire global navigation satellite system occultation sounder D (SGNOS-D) |
|  | 534 | PlanetiQ Pyxis GNSS occultation receiver A (Pyxis-A) |
|  | 535 | PlanetiQ Pyxis GNSS occultation receiver B (Pyxis-B) |
|  | 948 | Global positioning system occultation sensor (GPSOS) |
|  | 958 | Global navigation satellite system occultation sounder (GNOS) |

Table 3. Processing Mode and Disposition (New 5-bit Code Table)

| Descriptor | Value | Name |
| --- | --- | --- |
| 0 25 new | 0 | Operational (Near-Real-Time) |
|  | 1 | Operational (Non-Time-Critical) |
|  | 2 | Validation/Commissioning |
|  | 3 | Reprocessing |
|  | 4 | Testing/Development |

Table 4. Quality Flags for RO Data (16-bit Flag Table)

| Descriptor | Bit | Meaning when clear | Meaning when set |
| --- | --- | --- | --- |
| 0 33 039 | 1 | Nominal quality | Non-nominal quality |
|  | 2 | NRT product | Offline product |
|  | 3 | Descending occultation | Ascending occultation |
|  | 4 | Excess Phase processing nominal | Excess Phase processing non-nominal |
|  | 5 | Bending Angle processing nominal | Bending Angle processing non-nominal |
|  | 6 | Refractivity processing nominal | Refractivity processing non-nominal |
|  | 7 | Meteorological processing nominal | Meteorological processing non-nominal |
|  | 8 | Closed loop data only used | Open loop data included |
|  | 9 | No surface reflections detected | Surface reflections detected |
|  | 10 | L2P GNSS signals used | L2C GNSS signals used |
|  | 11–13 | Reserved | Reserved |
|  | 14 | Background profile nominal | Background profile non-nominal |
|  | 15 | Retrieved profile | Background profile |
|  | All 16 | As above | Missing |

Table 5. Satellite Classification (Code Table 002020)

| Descriptor | Value | Meaning |
| --- | --- | --- |
| 0 02 020 | 401 | GPS |
|  | 402 | GLONASS |
|  | 403 | GALILEO |
|  | 404 | BDS (BeiDou navigation satellite system) |
|  | 405 | QZSS (Quasi-Zenith satellite system) |

Table 6. Radio Occultation Diagnostic (New 5-bit Code Table)

| Descriptor | Value | Diagnostic type |
| --- | --- | --- |
| 0 08 new | 0 | Reserved |
|  | 1 | Local spectral width as a fraction of bending angle |
|  | 2 | Rényi entropy |
|  | 3 | Fractional bending angle difference - first asphericity component |
|  | 4 | Fractional bending angle difference - second asphericity component |
|  | 5 | Fractional refractivity difference - first asphericity component |
|  | 6 | Fractional refractivity difference - second asphericity component |
|  | 7 | Estimated systematic error as a fraction of bending angle |
|  | 8 | Estimated systematic error as a fraction of refractivity |
|  | 9 | Observation-to-background weighting ratio in refractivity retrieval |
|  | 31 | Missing value |

The above table contains examples of what could be in this new 5-bit code table. However, they are not meant as suggestions at this point. Individual entries should over time be proposed to WMO by users or data providers.

Table 7. Radio Occultation Vertical Significance (New 6-bit Code Table)

| Descriptor | Value | Vertical significance – how derived |
| --- | --- | --- |
| 0 08 new | 0 | Reserved |
|  | 1 | L2-signal extrapolation (impact height) |
|  | 2 | Geometric Optics/Wave Optics transition (impact height) |
|  | 3 | Open Loop/Closed Loop transition (straight line tangent altitude) |
|  | 4 | Ducting – largest negative bending angle gradient (impact height) |
|  | 5 | Ducting – another way (impact height) |
|  | 6 | Ducting – YAW (impact height) |
|  | 7 | Planetary boundary layer – large negative bending angle gradient (impact height) |
|  | 8 | Planetary boundary layer – large negative bending angle gradient (MSL altitude) |
|  | 9 | Planetary boundary layer – large negative refractivity gradient (MSL altitude) |
|  | 10 | Planetary boundary layer – large positive dry temperature gradient (MSL altitude) |
|  | 11 | Planetary boundary layer – local maximum in dry temperature (MSL altitude) |
|  | 12 | Tropopause – maximum covariance transform of bending angle (impact height) |
|  | 13 | Tropopause – maximum covariance transform of refractivity (MSL altitude) |
|  | 14 | Tropopause – local minimum in dry temperature (MSL altitude) |
|  | 15 | Tropopause – standard laps rate definition using dry temperature (MSL altitude) |
|  | 16 | Sporadic E layer – large scintillation in bending angle (impact height) |
|  | 17 | Sporadic E layer – YAW (MSL altitude) |
|  | 63 | Missing value |

The above table contains examples of what could be in this new 6-bit code table. However, they are not meant as suggestions at this point (YAW = yet another way). Individual entries should over time be proposed to WMO by users or data providers.