| Descriptor | Bit | Meaning when clear | Meaning when set |
| --- | --- | --- | --- |
| 0 33 039 | 1 | Nominal quality | Non-nominal quality |
|  | 2 | NRT product | Offline product |
|  | 3 | Descending (setting) occultation | Ascending (rising) occultation |
|  | 4 | Excess Phase processing nominal | Excess Phase processing non-nominal |
|  | 5 | Bending Angle processing nominal | Bending Angle processing non-nominal |
|  | 6 | Refractivity processing nominal | Refractivity processing non-nominal |
|  | 7 | Meteorological processing nominal | Meteorological processing non-nominal |
|  | 8 | Closed loop data only used | Open loop data included |
|  | 9 | No surface reflections detected | Surface reflections detected |
|  | 10 | L2P GNSS signals used | L2C GNSS signals used |
|  | 8–13 | Reserved | Reserved |
|  | 14 | Background profile nominal | Background profile non-nominal |
|  | 15 | Retrieved profile | Background profile |
|  | All 16 | As above | Missing |

Notes:

- Instead of or in addition to using bit 2, I would introduce a new (?) entry and Code Table describing the “Processing Mode and Disposition”. The following table lists what EUMETSAT uses for this:

| Descriptor | Value | Type/longname (shortname) |
| --- | --- | --- |
| x xx xxx | 0 | Operational (Near-Real-Time) |
|  | 1 | Operational (Non-Time-Critical) |
|  | 2 | Validation/Commissioning |
|  | 3 | Reprocessing |
|  | 4 | Backlog |
|  | 5 | Global |
|  | 6 | Regional |
|  | 7 | Local |
|  | 8 | Development |
|  | 9 | Testing |

- If we keep flags for the successful completion of excess phase, bending angle and refractivity processing, one could also add a flag describing the successful POD processing. However: If POD goes wrong, excess phase goes wrong; if excess phase goes wrong, bending angle goes wrong etc. On the other hand, there seems to be a preference of NWP centres to not get any degraded data as bufr. In that case, these flags will never be set in products disseminated to users, and their information content is effectively zero…

- Closed vs. open loop data: Most modern receivers use open loop data everywhere. If receivers use both, a more interesting quantity would be where the transition between closed and open loop takes places. Only some very old receivers (GPS/MET, CHAMP, GRACE, maybe more?) exclusively use closed loop tracking.

- I’m not aware of anyone actively flagging surface reflections.

- L2P vs. L2C: When introduced in GPS and first exploited by COSMIC-1, this was exciting because the new civil signal promised higher SNRs. Nowadays, many modern receivers are not capable of doing codeless tracking (as required for the encrypted L2P signals), and encrypted legacy GPS signals will disappear at the end of the 2020’s. Instead, modern receivers increasingly make use of pilot signals, i.e. signals without any data modulation. This cannot be mapped with a single (or a few) binary flags; using a new entry similar to the frequencies together with a code table identifying the signals being exploited by the instrument would be better. One could stick to RINEX conventions; IGS lists 109 different signal types across the major GNSS systems (GPS, Galileo, GLONASS, Beidou, QZSS, IRSS) in their RINEX 3.05 specification. Or instead of a code table, use a 3-char text field…

- If we follow Stig’s suggestion to remove level 2 products (Temperature, moisture and surface pressure), we can also drop bits 7, 14 and 15.