Short sequences

| Data Field | Element Name | Descrip. | Table B Scale | Table B Ref. Val. | Table B Width | Units | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1–4 | WIGOS identifier | 3 01 150 |  |  |  |  |  |
| 1 | Identifier series | 0 01 125 | 0 | 0 | 4 | Numeric |  |
| 2 | Issuer of identifier | 0 01 126 | 0 | 0 | 16 | Numeric |  |
| 3 | Issue number | 0 01 127 | 0 | 0 | 16 | Numeric |  |
| 4 | Local identifier | 0 01 128 | 0 | 0 | 128 | Character |  |

| Data Field | Element Name | Descrip. | Table B Scale | Table B Ref. Val. | Table B Width | Units | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1–3 | Satellite data introducer | New |  |  |  |  | To be proposed by this BUFR discussion group – we need a new name for it since ‘Satellite data introducer’ is already taken |
| 1 | Satellite Identifier | 0 01 007 | 0 | 0 | 10 | Code Table | LEO or LEO constellation – See Table 1 |
| 2 | Satellite sub-identifier | 0 01 016 | 0 | 0 | 16 | Numeric | Satellite ID in LEO constellation (missing if not applicable) |
| 3 | Satellite instrument | 0 02 019 | 0 | 0 | 11 | Code Table | e.g. 202 = GRAS – See Table 2 |

| Data Field | Element Name | Descrip. | Table B Scale | Table B Ref. Val. | Table B Width | Units | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1–3 | Aircraft data introducer | New |  |  |  |  | To be proposed by the first provider of such data, e.g., Scripps Institute of Oceanography – the below is tentative until then |
| 1 | Aircraft registration number or other identification | 0 01 008 | 0 | 0 | 64 | Character |  |
| 2 | Aircraft tail number | 0 01 110 | 0 | 0 | 48 | Code Table |  |
| 3 | Upper air remote sensing instrument type | 0 02 006 | 0 | 0 | 6 | Code Table |  |

| Data Field | Element Name | Descrip. | Table B Scale | Table B Ref. Val. | Table B Width | Units | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1–3 | Balloon data introducer | New |  |  |  |  | To be proposed by the first provider of such data, e.g., Scripps Institute of Oceanography – the below is tentative until then |
| 1 | Balloon Registration number or other identification | New | 0 | 0 | 80 | Character | To be proposed by Météo-France Research Department (might be redundant with WIGOS issue number – follow the progress at https://github.com/wmo-im/BUFR4/issues/208) |
| 2 | Balloon flight number | New | 0 | 0 | 64 | Character | To be proposed by Météo-France Research Department (might be redundant with WIGOS local identifier – follow the progress at https://github.com/wmo-im/BUFR4/issues/208) |
| 3 | Upper air remote sensing instrument type | 0 02 006 | 0 | 0 | 6 | Code Table |  |

| Data Field | Element Name | Descrip. | Table B Scale | Table B Ref. Val. | Table B Width | Units | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1–2 | Station data introducer | New |  |  |  |  | To be proposed by the first provider of such data – the below is tentative until then |
| 1 | Short station or site name | 0 01 018 | 0 | 0 | 40 | Character |  |
| 2 | Upper air remote sensing instrument type | 0 02 006 | 0 | 0 | 6 | Code Table |  |

BUFR Section 3 (Data Description)

| Octet number | Contents | Value for RO |
| --- | --- | --- |
| 1–3 | Length of Section 3 (octets) | 13 or 14 |
| 4 | Reserved | 0 |
| 5–6 | Number of datasets (observations) | Integer > 0 |
| 7 | Section 4 Data flags: Bit 1 = 1 for Observed data, 0 for Other data Bit 2 = 1 for Compressed, 0 for Uncompressed Bits 3–8: Reserved (set to 0) | 128 (Observed, Uncompressed) or 192 (Observed, Compressed) |
| 8–9 | Descriptor (WIGOS identifier) | 49558 (3 x 16384 + 1 x 256 + 150) |
| 10–11 | Descriptor (e.g., new Aircraft data introducer) | New (3 x 16384 + X x 256 + Y) |
| 12–13 | Descriptor (new RO BUFR template) | New (3 x 16384 + 10 x 256 + Y) |
| [14–] | Repeat 2-octets per descriptor Total number of descriptors = (Section length - 7) / 2 |  |
| [14] | Pad byte (set to 0) – optional for Edition 4 | 0 (if Section length = 14) |

New RO BUFR template

| Data Field | Element Name | Descrip. | Table B Scale | Table B Ref. Val. | Table B Width | Units | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Radio Occultation header |  |  |  |  |  |  |  |
| 1 | Originating centre | 0 01 035 | 0 | 0 | 16 | Code Table | e.g. 94 = Copenhagen (DMI) |
| 2 | Originating sub-centre | 0 01 034 | 0 | 0 | 8 | Code Table | e.g. 220 = NOAA |
| 3 | Product type | 0 02 172 | 0 | 0 | 8 | Code Table | 3 = occultation sounding |
| 4 | Software ID and version | 0 25 061 | 0 | 0 | 96 | Character | Identifies processing algorithms used by org. centre |
| 5 | Processing mode and disposition | 0 25 new | 0 | 0 | 5 | Code Table | e.g. 3 = Reprocessing – See Table 3 |
| Reference time |  |  |  |  |  |  |  |
| 6 | Time Significance | 0 08 021 | 0 | 0 | 5 | Code Table | 17 = start of phenomenon or 25 = nominal reporting time |
| 7–9 | Date | 3 01 011 |  |  |  |  |  |
| 7 | Year | 0 04 001 | 0 | 0 | 12 | Year | 4-digit year |
| 8 | Month | 0 04 002 | 0 | 0 | 4 | Month |  |
| 9 | Day | 0 04 003 | 0 | 0 | 6 | Day |  |
| 10–11 | Time | 3 01 012 |  |  |  |  |  |
| 10 | Hour | 0 04 004 | 0 | 0 | 5 | Hour |  |
| 11 | Minute | 0 04 005 | 0 | 0 | 6 | Minute |  |
|  | Change Table B bit width | 2 01 138 |  |  |  |  | Add 10 to bit width |
|  | Change Table B scale | 2 02 131 |  |  |  |  | Add 3 to scale |
| 12 | Second | 0 04 006 | (3) 0 | 0 | (16) 6 | Second | 0.000 – 59.999s to 1ms |
|  | Change scale to Table B | 2 02 000 |  |  |  |  |  |
|  | Change width to Table B | 2 01 000 |  |  |  |  |  |
| RO summary quality information |  |  |  |  |  |  |  |
| 13 | Quality flags for RO Data | 0 33 new | 0 | 0 | 31 | Flag table | See old bit-flags in Table 4 |
| 14 | Percent confidence | 0 33 007 | 0 | 0 | 7 | % | 0 = bad, 100 = good |
| LEO & GNSS POD at reference time |  |  |  |  |  |  |  |
| 15 | Reference system | 0 01 150 | 0 | 0 | 16 | Code Table | 5 = ECEF or ECR, right-handed |
| 16–18 | Location of Platform (see Note 10) | 3 04 030 |  |  |  |  | LEO, to 1cm |
| 16 | Distance from Earth’s centre in direction 0o longitude | 0 27 031 | 2 | -1073741824 | 31 | Metres | (X) |
| 17 | Distance from Earth’s centre in direction 90o east longitude | 0 28 031 | 2 | -1073741824 | 31 | Metres | (Y) |
| 18 | Distance from Earth’s centre in direction of North Pole | 0 10 031 | 2 | -1073741824 | 31 | Metres | (Z) |
| 19–21 | Velocity of Platform (see Note 10) | 3 04 031 |  |  |  |  | LEO, to 10mm/s |
| 19 | Absolute platform velocity – first component. | 0 01 041 | 5 | -1073741824 | 31 | Metres per second | (X) |
| 20 | Absolute platform velocity – second component. | 0 01 042 | 5 | -1073741824 | 31 | Metres per second | (Y) |
| 21 | Absolute platform velocity – third component. | 0 01 043 | 5 | -1073741824 | 31 | Metres per second | (Z) |
| 22 | Satellite Classification | 0 02 020 | 0 | 0 | 9 | Code Table | GNSS series, e.g. 401=GPS – See Table 5 |
| 23 | Platform Transmitter ID | 0 01 050 | 0 | 0 | 17 | Numeric | GNSS PRN or similar |
|  | Change Table B scale | 2 02 127 |  |  |  |  | Subtract 1 from scale |
| 24–26 | Location of Platform (see Note 10) | 3 04 030 |  |  |  |  | GNSS, to 10cm |
| 24 | Distance from Earth’s centre in direction 0o longitude | 0 27 031 | (1) 2 | -1073741824 | 31 | Metres | (X) |
| 25 | Distance from Earth’s centre in direction 90o east longitude | 0 28 031 | (1) 2 | -1073741824 | 31 | Metres | (Y) |
| 26 | Distance from Earth’s centre in direction of North Pole | 0 10 031 | (1) 2 | -1073741824 | 31 | Metres | (Z) |
|  | Change scale to Table B | 2 02 000 |  |  |  |  |  |
| 27–29 | Velocity of Platform (see Note 10) | 3 04 031 |  |  |  |  | GNSS, to 10mm/s |
| 27 | Absolute platform velocity – first component. | 0 01 041 | 5 | -1073741824 | 31 | Metres per second | (X) |
| 28 | Absolute platform velocity – second component. | 0 01 042 | 5 | -1073741824 | 31 | Metres per second | (Y) |
| 29 | Absolute platform velocity – third component. | 0 01 043 | 5 | -1073741824 | 31 | Metres per second | (Z) |
| Local Earth parameters at reference time |  |  |  |  |  |  |  |
| 30 | Straight line tangent altitude | 0 10 new | 0 | -400000 | 19 | Metres | -400 – 124km wrt WGS-84 |
| 31–32 | Location (high accuracy) | 3 01 021 |  |  |  |  |  |
| 31 | Latitude | 0 05 001 | 5 | -9000000 | 25 | Degrees | to 10-5 deg (~1m) wrt WGS-84 |
| 32 | Longitude | 0 06 001 | 5 | -18000000 | 26 | Degrees | to 10-5 deg (~1m) wrt WGS-84 |
| 33–35 | Location of point | 3 04 030 |  |  |  |  | (X, Y, Z) centre of curvature to 1cm |
| 33 | Distance from Earth’s centre in direction 0o longitude | 0 27 031 | 2 | -1073741824 | 31 | Metres | (X) |
| 34 | Distance from Earth’s centre in direction 90o east longitude | 0 28 031 | 2 | -1073741824 | 31 | Metres | (Y) |
| 35 | Distance from Earth’s centre in direction of North pole | 0 10 031 | 2 | -1073741824 | 31 | Metres | (Z) |
| 36 | Earth’s local radius of curvature | 0 10 035 | 1 | 62 000 000 | 22 | Metres | RoC of WGS-84 ellipsoid. 6200–6600 km to 10cm |
| 37 | Bearing or azimuth | 0 05 021 | 2 | 0 | 16 | Deg. True | GNSS-to-LEO line of sight |
| 38 | Geoid undulation | 0 10 036 | 2 | -15 000 | 15 | Metres | Geoid height above WGS-84 ellipsoid. ±150m to 1cm |
| Time period of occultation |  |  |  |  |  |  |  |
| 39 | Time period (between reference time and start of occultation) | 0 04 026 | 0 | -4096 | 13 | Second | -4096s – 4096s (normally negative) |
| 40 | Time period (between reference time and end of occultation) | 0 04 026 | 0 | -4096 | 13 | Second | -4096s – 4096s (normally positive) |
| Diagnostic quality information |  |  |  |  |  |  |  |
|  | Delayed Replication | 1 06 000 |  |  |  |  | Delayed replication of next 6 descriptors |
| 41 | Replication factor | 0 31 001 | 0 | 0 | 8 | Numeric | Number of height ranges, n0 |
| 42 | Height (bottom of range) | 0 07 007 | 0 | -1000 | 17 | Metres | -1km – 130km, to 1m |
| 43 | Height (top of range) | 0 07 007 | 0 | -1000 | 17 | Metres | -1km – 130km, to 1m |
|  | Change Table B scale | 2 02 134 |  |  |  |  | Add 6 to scale |
| 44 | Satellite channel centre frequency | 0 02 153 | (-2) -8 | 0 | 26 | Hz | 0 – 6.7GHz, to 0.1kHz |
|  | Change scale to Table B | 2 02 000 |  |  |  |  |  |
| 45 | Signal-to-noise ratio, mean value over specified height range | 0 33 new | 1 | 0 | 20 | Numeric | 0 – 100000, to 0.1 |
| N0 | (end delayed replication – number of height ranges) |  |  |  |  |  | N0 = 41+4.n0 elements |
|  | Delayed Replication | 1 06 000 |  |  |  |  | Delayed replication of next 6 descriptors |
| N0+1 | Replication factor | 0 31 001 | 0 | 0 | 8 | Numeric | Number of height ranges, n1 |
| N0+2 | Height (bottom of range) | 0 07 007 | 0 | -1000 | 17 | Metres | -1km – 130km, to 1m |
| N0+3 | Height (top of range) | 0 07 007 | 0 | -1000 | 17 | Metres | -1km – 130km, to 1m |
|  | Change Table B scale | 2 02 134 |  |  |  |  | Add 6 to scale |
| N0+4 | Satellite channel centre frequency | 0 02 153 | (-2) -8 | 0 | 26 | Hz | 0 – 6.7GHz, to 0.1kHz, Ionosphere corrected = 0 |
|  | Change scale to Table B | 2 02 000 |  |  |  |  |  |
| N0+5 | Bending angle noise, standard deviation over specified height range | 0 33 new | 8 | 0 | 16 | Radians | 0 – 6x10-4 rad, to 10-8 rad |
| N1 | (end delayed replication – number of height ranges) |  |  |  |  |  | N1 = N0+1+4.n1 elements |
| N1+1 | Satellite channel centre frequency | 0 02 153 | -8 | 0 | 26 | Hz | Missing = cancel previous value |
| N1+2 | Height | 0 07 007 | 0 | -1000 | 17 | Metres | Missing = cancel previous value |
| Characteristic heights |  |  |  |  |  |  |  |
|  | Delayed Replication | 1 03 000 |  |  |  |  | Delayed replication of next 3 descriptors |
| N1+3 | Replication factor | 0 31 001 | 0 | 0 | 8 | Numeric | Number of characteristic heights, n2 |
| N1+4 | Radio occultation vertical significance | 0 08 new | 0 | 0 | 5 | Code Table | See Table 7 |
| N1+5 | Height | 0 10 007 | 0 | -1000 | 17 | Metres | -1km – 130km, to 1m |
| N1+6 | Standard deviation altitude | 0 10 050 | 2 | 0 | 16 | Metres | 0 – 655m, to 1cm |
| N2 | (end delayed replication – number of characteristic heights) |  |  |  |  |  | N2 = N1+3+3.n2 elements |
| N2+1 | Radio occultation vertical significance | 0 08 new | 0 | 0 | 5 | Code Table | Missing = cancel previous value |
| RO 'Step 1b' data (see Notes 2 & 5) |  |  |  |  |  |  |  |
|  | Delayed replication | 1 25 000 |  |  |  |  | Delayed replication of next 25 descriptors |
| N2+2 | Replication factor | 0 31 002 | 0 | 0 | 16 | Numeric | Number of Step 1b samples, n6 |
| N2+3–N2+4 | Location (high accuracy) | 3 01 021 |  |  |  |  |  |
| N2+3 | Latitude | 0 05 001 | 5 | -9000000 | 25 | Degrees | to 10-5 deg (~1m) wrt WGS-84 |
| N2+4 | Longitude | 0 06 001 | 5 | -18000000 | 26 | Degrees | to 10-5 deg (~1m) wrt WGS-84 |
| N2+5 | Bearing or azimuth | 0 05 021 | 2 | 0 | 16 | Deg. True | GNSS-to-LEO line of sight |
| N2+6 | Impact Parameter | 0 07 040 | 1 | 62 000 000 | 22 | Metres | 6200–6600 km to 10cm (distance from centre of curvature) |
|  | Delayed Replication | 1 11 000 |  |  |  |  | Delayed replication of next 11 descriptors |
| N2+7 | Replication factor | 0 31 001 | 0 | 0 | 8 | Numeric | Number of frequencies, n4 |
|  | Change Table B scale | 2 02 134 |  |  |  |  | Add 6 to scale |
| N2+8 | Satellite channel centre frequency | 0 02 153 | (-2) -8 | 0 | 26 | Hz | 0 – 6.7GHz, to 0.1kHz, Ionosphere corrected = 0 |
|  | Change scale to Table B | 2 02 000 |  |  |  |  |  |
| N2+9 | Signal type | 0 02 new | 0 | 0 | 24 | Character | Three-letter code, RINEX convention |
| N2+10 | Bending Angle | 0 15 037 | 8 | -100 000 | 23 | Radians | -10-3 – 8x10-2 rad to 10-8 rad |
| N2+11 | Estimated error in bending angle | 0 15 new | 8 | 0 | 20 | Radians | 0 – 10-2 rad to 10-8 rad |
| N2+12 | Calibrated phase difference | 0 15 new | 3 | -500 | 10 | Metres | -50cm – 50cm to 1mm |
| N2+13 | Estimated error in calibrated phase difference | 0 15 new | 3 | 0 | 7 | Metres | 0 – 12cm to 1mm |
|  | Delayed Replication | 1 01 000 |  |  |  |  | Delayed replication of next descriptor |
| N2+14 | Short delayed replication factor | 0 31 000 | 0 | 0 | 1 | Numeric | Indicate presence of aircraft or station RO data, n3 |
| N2+15 – N2+18 | Aircraft or station RO data | 3 12 new |  |  |  |  |  |
|  | Increase Table B scale, reference value and bit width (all at once) | 2 07 001 |  |  |  |  | Add 1 to scale, multiply reference value by 10, and add 4 to bit width |
| N2+15 | Height (of RO antenna) | 0 10 007 | (1) 0 | (-10000) -1000 | (21) 17 | Metres | Geom. altitude, -1km – 200km, to 10cm, wrt Geoid (MSL) |
|  | Change scale, reference value and bit width to Table B | 2 07 000 |  |  |  |  |  |
| N2+16 | Atmospheric refractivity (in-situ at RO antenna) | 0 15 036 | 3 | 0 | 19 | N-units | 0 – 500 to 10-3 N-units |
| N2+17 | Bending angle (partial) | 0 15 037 | 8 | -100 000 | 23 | Radians | -10-3 – 8x10-2 rad to 10-8 rad |
| N2+18 | Estimated error in bending angle | 0 15 new | 8 | 0 | 20 | Radians | 0 – 10-2 rad to 10-8 rad |
| N3 | (end delayed replication – presence of aircraft or station RO data) |  |  |  |  |  | N3 = N2+14+4.n3 elements |
| N4 | (end delayed replication – number of frequencies) |  |  |  |  |  | N4 = N2+7+(7+4.n3).n4 elements |
| N4+1 | Signal type | 0 02 new | 0 | 0 | 24 | Character | Missing = cancel previous value |
| N4+2 | Satellite channel centre frequency | 0 02 153 | -8 | 0 | 26 | Hz | 0 = Ionosphere corrected |
| N4+3 | Percent confidence | 0 33 007 | 0 | 0 | 7 | % | 0 = bad, 100 = good |
|  | Delayed Replication | 1 05 000 |  |  |  |  | Delayed replication of next 5 descriptors |
| N4+4 | Replication factor | 0 31 001 | 0 | 0 | 8 | Numeric | Number of diagnostic entries, n5 |
| N4+5 | Radio occultation Diagnostic type | 0 08 new | 0 | 0 | 5 | Code Table | See Table 6 |
| N4+6 | Decimal scale of following significands | 0 08 090 | 0 | -127 | 8 | Numeric | Decimal scale (see next descriptor) |
| N4+7 | Scaled diagnostic | 0 33 new | 0 | -131 071 | 18 | Numeric | Scaled value (actual value = scaled value x 10decimal scale) |
| N4+8 | Decimal scale of following significands | 0 08 090 | 0 | -127 | 8 | Numeric | Missing = cancel previous value |
| N4+9 | Radio occultation Diagnostic type | 0 08 new | 0 | 0 | 5 | Code Table | Missing = cancel previous value |
| N5 | (end delayed replication – number of diagnostic entries) |  |  |  |  |  | N5 = N4+4+5.n5 elements |
| N6 | (end delayed replication – number of step 1B samples) |  |  |  |  |  | N6 = N2+2+(9+(7+4.n3).n4+5.n5).n6 elements |
| N6+1 | Impact Parameter | 0 07 040 | 1 | 62 000 000 | 22 | Metres | Missing = cancel previous value |
| N6+2 | Bearing or azimuth | 0 05 021 | 2 | 0 | 16 | Deg. True | Missing = cancel previous value |
| N6+3–N6+4 | Location (high accuracy) | 3 01 021 |  |  |  |  |  |
| N6+3 | Latitude | 0 05 001 | 5 | -9000000 | 25 | Degrees | Missing = cancel previous value |
| N6+4 | Longitude | 0 06 001 | 5 | -18000000 | 26 | Degrees | Missing = cancel previous value |
| RO 'Step 2a' data (see Notes 3–5 & 11) |  |  |  |  |  |  |  |
|  | Delayed replication | 1 15 000 |  |  |  |  | Delayed replication of next 15 descriptors |
| N6+5 | Replication factor | 0 31 002 | 0 | 0 | 16 | Numeric | Number of Step 2a samples, n8 |
|  | Increase Table B scale, reference value and bit width (all at once) | 2 07 001 |  |  |  |  | Add 1 to scale, multiply reference value by 10, and add 4 to bit width |
| N6+6 | Height | 0 07 007 | (1) 0 | (-10000) -1000 | (21) 17 | Metres | Geom. altitude, -1km – 200km, to 10cm, wrt Geoid (MSL) |
| N6+7 | Geopotential height (see Note 11) | 0 10 009 | (1) 0 | (-10000) -1000 | (21) 17 | gpm | Geopot. altitude, -1km – 200km, to 10cm, wrt Geoid (MSL) |
| N6+8 | Atmospheric refractivity | 0 15 036 | (4) 3 | 0 | (23) 19 | N-units | 0 – 800, to 10-4 N-units |
| N6+9 | Estimated error in atmospheric refractivity | 0 15 new | (4) 3 | 0 | (20) 16 | N-units | 0 – 100, to 10-4 N-units |
| N6+10 | Dry temperature | 0 12 new | (2) 1 | 0 | (16) 12 | K | 0 – 600K, to 0.01 K |
|  | Change scale, reference value and bit width to Table B | 2 07 000 |  |  |  |  |  |
| N6+11 | Percent confidence | 0 33 007 | 0 | 0 | 7 | % | 0 = bad, 100 = good |
|  | Delayed Replication | 1 04 000 |  |  |  |  | Delayed replication of next 4 descriptors |
| N6+12 | Replication factor | 0 31 001 | 0 | 0 | 8 | Numeric | Number of diagnostic entries, n7 |
| N6+13 | Radio occultation Diagnostic type | 0 08 new | 0 | 0 | 5 | Code Table | See Table 6 |
| N6+14 | Decimal scale of following significands | 0 08 090 | 0 | -127 | 8 | Numeric | Decimal scale (see next descriptor) |
| N6+15 | Scaled diagnostic | 0 33 new | 1 | -131 071 | 18 | Numeric | Scaled value (actual value = scaled value x 10decimal scale) |
| N6+16 | Decimal scale of following significands | 0 08 090 | 0 | -127 | 8 | Numeric | Missing = cancel previous value |
| N6+17 | Radio occultation Diagnostic type | 0 08 new | 0 | 0 | 5 | Code Table | Missing = cancel previous value |
| N7 | (end delayed replication – number of diagnostics entries) |  |  |  |  |  | N7 = N6+12+5.n7 elements |
| N8 | (end delayed replication – number of step 2a samples) |  |  |  |  |  | N8 = N6+5+(7+5.n7).n8 elements |
| Blue entries indicate new or changed elements. |  |  |  |  |  |  |  |

NOTES:

- Where parameter value ranges are given in the Comments column, these are the nominal User Requirements; the actual values allowed by the descriptor may be different. The quoted range is guaranteed to be encoded correctly. (E.g. surface pressure range is quoted as 250-1100 hPa, but the descriptor can encode any value between 0.0 and 1638.2 hPa)

- For GTS dissemination, Step 1b data would be sampled, interpolated and/or smoothed to a smaller number of profile levels (for GRAS, 247 fixed levels from near surface to 60 km) (n1 samples; typically 200–300). There should be a minimum of one pair of impact parameter and bending angles per sample (n0 ≥ 1), representing ionosphere-corrected values (when mean frequency is set to zero); sets for GPS L1 and L2 signals may also be present, and the delayed replication mechanism could in principle be used to include sets derived from additional (future) frequencies.

- For GTS dissemination, Step 2a data would be sampled, interpolated and/or smoothed to a smaller number of profile levels (for GRAS, 247 fixed levels from near surface to 60 km) (n2 samples; typically 100–300). Depending on the processing algorithms, Step 2a profiles may or may not be on the same height grid as Step 1b.

- Step 2b data are sampled, interpolated and/or smoothed to a smaller number of profile levels (nominally surface to 60 km with an average spacing of 1 km) (n3 samples; typically 50–150). For 1D-VAR retrievals, the levels would usually be those of the a priori background, but could be on the same height grid as the Step 2a profile.

- All profile data should be ordered in increasing altitude, whether the original occultation was ascending or descending.

- A Step 2 profile can be assumed to be vertical and at the nominal time and surface location of the given local radius of curvature co-ordinates. Alternatively, the height information can be converted to impact parameter, then Step 1b latitudes & longitudes can be interpolated to the Step 2 samples to give slant profiles.

- The total number of data elements is 47 + n1x(5+n0x6) + n2x6 + n3x10 (currently in new template: 52+4.n0+4.n1+3.n2+(9+(7+4.n3).n4+5.n5).n6+(7+5.n7).n8). For a nominal message containing 200 Step 1b (L1+L2+Corrected) and 150 Step 2a and 100 Step 2b profile samples, this would total 6547 elements.

- A single BUFR message contains one (uncompressed) occultation; each message is 11,338 bytes long for a nominal message containing 200 Step 1b (L1+L2+Corrected), 150 Step 2a and 100 Step 2b profile samples. Indicative message lengths for other combinations of samples are shown in Table 11.

- RO instruments can typically produce up to about 500 occultations (BUFR messages) per day per satellite; hence the maximum RO data volume in BUFR would be of the order of 7 Mbytes per day per receiver.

- The components of satellite location and absolute satellite velocity are defined in an Earth-centered inertial frame at the nominal time with the following orientation:

(X) First component: along the line from the Earth's centre to 0° longitude at the Equator.

(Y) Second component: along the line from the Earth's centre to 90° east longitude at the Equator.

(Z) Third component: along the line from the Earth's centre to the North Pole.

See also the WMO FM 94 BUFR Table B document, class 01, footnote (6), and BUFR descriptors 0 27 031, 0 28 031 and 0 10 031.

Table 1. GNSS RO Satellite Identifier (Common Code Table C–5)

| Descriptor | Value | Name | Value | Name |
| --- | --- | --- | --- | --- |
| 0 01 007 | 3 | Metop-1 (Metop-B) | 740 | COSMIC-1 |
|  | 4 | Metop-2 (Metop-A) | 741 | COSMIC-2 |
|  | 5 | Metop-3 (Metop-C) | 742 | COSMIC-3 |
|  | 40 | Ørsted (or Oersted) | 743 | COSMIC-4 |
|  | 41 | CHAMP | 744 | COSMIC-5 |
|  | 42 | TerraSAR-X | 745 | COSMIC-6 |
|  | 43 | TanDEM-X | 750 | COSMIC-2 E1 |
|  | 44 | PAZ | 751 | COSMIC-2 E2 |
|  | 66 | Sentinel-6A | 752 | COSMIC-2 E3 |
|  | 67 | Sentinel-6B | 753 | COSMIC-2 E4 |
|  | 265 | GeoOptics CICERO OP1 | 754 | COSMIC-2 E5 |
|  | 266 | GeoOptics CICERO OP2 | 755 | COSMIC-2 E6 |
|  | 267 | PlanetiQ GNOMES-A | 786 | C/NOFS |
|  | 268 | PlanetiQ GNOMES-B | 800 | SUNSAT |
|  | 269 | Spire Lemur 3U CubeSat | 803 | GRACE C (GRACE-FO) |
|  | 421 | Oceansat-2 | 804 | GRACE D (GRACE-FO) |
|  | 440 | Megha-Tropiques | 820 | SAC-C |
|  | 522 | FY-3C | 825 | KOMPSAT-5 |
|  | 523 | FY-3D |  |  |
|  | 722 | GRACE A |  |  |
|  | 723 | GRACE B |  |  |

Table 2. GNSS RO Satellite Instrument Identifier (Common Code Table C–8)

| Descriptor | Value | Type/longname (shortname) |
| --- | --- | --- |
| 0 02 019 | 98 | Atmospheric temperature and humidity sounder (GPS receiver) |
|  | 102 | GPS turborogue space receiver (TRSR) (CHAMP GPS sounder) |
|  | 103 | Integrated GPS and Occultation Receiver (IGOR) |
|  | 104 | Triple-G (GPS, Galileo, GLONASS) (Tri-G) |
|  | 202 | GNSS receiver for atmospheric sounding (GRAS) |
|  | 287 | Radio Occultation Sounder of the Atmosphere (ROSA) |
|  | 351 | GPS demonstration receiver (GPSDR) |
|  | 526 | GeoOptics Cion GNSS occultation receiver A (Cion-A) |
|  | 527 | GeoOptics Cion GNSS occultation receiver B (Cion-B) |
|  | 528 | GeoOptics Cion GNSS occultation receiver C (Cion-C) |
|  | 529 | GeoOptics Cion GNSS occultation receiver D (Cion-D) |
|  | 530 | Spire global navigation satellite system occultation sounder A (SGNOS-A) |
|  | 531 | Spire global navigation satellite system occultation sounder B (SGNOS-B) |
|  | 532 | Spire global navigation satellite system occultation sounder C (SGNOS-C) |
|  | 533 | Spire global navigation satellite system occultation sounder D (SGNOS-D) |
|  | 534 | PlanetiQ Pyxis GNSS occultation receiver A (Pyxis-A) |
|  | 535 | PlanetiQ Pyxis GNSS occultation receiver B (Pyxis-B) |
|  | 948 | Global positioning system occultation sensor (GPSOS) |
|  | 958 | Global navigation satellite system occultation sounder (GNOS) |

Table 3. Processing Mode and Disposition (New 5-bit Code Table)

| Descriptor | Value | Name |
| --- | --- | --- |
| 0 25 new | 0 | Operational (Near-Real-Time) |
|  | 1 | Operational (Non-Time-Critical) |
|  | 2 | Validation/Commissioning |
|  | 3 | Reprocessing |
|  | 4 | Testing/Development |

Table 4. Quality Flags for RO Data (16-bit Flag Table)

| Descriptor | Bit | Meaning when clear | Meaning when set |
| --- | --- | --- | --- |
| 0 33 039 | 1 | Nominal quality | Non-nominal quality |
|  | 2 | NRT product | Offline product |
|  | 3 | Descending occultation | Ascending occultation |
|  | 4 | Excess Phase processing nominal | Excess Phase processing non-nominal |
|  | 5 | Bending Angle processing nominal | Bending Angle processing non-nominal |
|  | 6 | Refractivity processing nominal | Refractivity processing non-nominal |
|  | 7 | Meteorological processing nominal | Meteorological processing non-nominal |
|  | 8–14 | Reserved | Reserved |
|  | 15 | Retrieved profile | Background profile |
|  | All 16 | As above | Missing |

Table 5. Satellite Classification (Code Table 002020)

| Descriptor | Value | Meaning |
| --- | --- | --- |
| 0 02 020 | 401 | GPS |
|  | 402 | GLONASS |
|  | 403 | GALILEO |
|  | 404 | BDS (BeiDou navigation satellite system) |
|  | 405 | QZSS (Quasi-Zenith satellite system) |

Table 6. Radio Occultation Diagnostic (New 5-bit Code Table)

| Descriptor | Value | Diagnostic type |
| --- | --- | --- |
| 0 08 new | 1 | Local spectral width as a fraction of bending angle |
|  | 2 | Rényi entropy |
|  | 3 | Fractional bending angle difference - first asphericity component |
|  | 4 | Fractional bending angle difference - second asphericity component |
|  | 5 | Fractional refractivity difference - first asphericity component |
|  | 6 | Fractional refractivity difference - second asphericity component |
|  | 7 | Estimated systematic error as a fraction of bending angle |
|  | 8 | Estimated systematic error as a fraction of refractivity |
|  | 9 | Observation-to-background weighting ratio in refractivity retrieval |

Table 7. Radio Occultation Vertical Significance (New 5-bit Code Table)

| Descriptor | Value | Vertical significance – how derived |
| --- | --- | --- |
| 0 08 new | 0 | Reserved |
|  | 1 | L2-signal extrapolation (impact height) |
|  | 2 | Geometric Optics/Wave Optics transition (impact height) |
|  | 3 | Open Loop/Closed Loop transition (straight line tangent altitude) |
|  | 4 | Ducting – largest negative bending angle gradient (impact height) |
|  | 5 | Ducting – another way (impact height) |
|  | 6 | Ducting – YAW (impact height) |
|  | 7 | Planetary boundary layer – large negative bending angle gradient (impact height) |
|  | 8 | Planetary boundary layer – large negative bending angle gradient (MSL altitude) |
|  | 9 | Planetary boundary layer – large negative refractivity gradient (MSL altitude) |
|  | 10 | Planetary boundary layer – large positive dry temperature gradient (MSL altitude) |
|  | 11 | Planetary boundary layer – local maximum in dry temperature (MSL altitude) |
|  | 12 | Tropopause – maximum covariance transform of bending angle (impact height) |
|  | 13 | Tropopause – maximum covariance transform of refractivity (MSL altitude) |
|  | 14 | Tropopause – local minimum in dry temperature (MSL altitude) |
|  | 15 | Tropopause – standard laps rate definition using dry temperature (MSL altitude) |
|  | 16 | Sporadic E layer – large scintillation in bending angle (impact height) |
|  | 17 | Sporadic E layer – YAW (MSL altitude) |
|  | 31 | Missing value |